EVSE PROJECT

Electric Vehicle Supply Equipment

Full Design & Implementation Documentation

Hardware Platform STM32F103C8 · ESP32-C6 IEC 61851-1 · SAE J1772 · OCPP 1.6J

Version 1.0  |  Personal Build Project

# Table of Contents

Table of Contents	1

1. Introduction	1

2. System Overview	1

2.1  Functional Block Diagram	1

2.2  Board Responsibilities Summary	1

3. Standards & Working Principle	1

3.1  Applicable Standards	1

3.2  The IEC 61851 / J1772 Pilot Signal — In Depth	1

3.2.1  Signal Characteristics	1

3.2.2  Current Advertisement via PWM Duty Cycle	1

3.3  Power Path	1

4. Hardware Design	1

4.1  Bill of Materials	1

4.2  Pilot Signal Conditioning Circuit	1

4.3  Current Sensing Circuit	1

4.4  STM32 Pin Assignment	1

4.5  ESP32-C6 Pin Assignment	1

5. STM32F103C8 Firmware	1

5.1  Toolchain Setup	1

5.2  Pilot PWM Configuration	1

5.3  Pilot State Machine	1

5.4  Fault Detection	1

5.5  FreeRTOS Task Layout	1

5.6  UART Protocol (STM32 → ESP32)	1

6. ESP32-C6 Firmware	1

6.1  Development Environment	1

6.2  FreeRTOS Task Layout	1

6.3  OCPP 1.6J Integration	1

6.4  BLE Authorisation	1

6.5  Local Web Dashboard	1

7. Step-by-Step Implementation	1

Phase 1 — Bench Validation (No Mains)	1

Phase 2 — Power Path Integration (Low Voltage First)	1

Phase 3 — Full Integration (Qualified Electrician Required)	1

Phase 4 — System Hardening	1

8. Testing & Validation	1

8.1  Test Matrix	1

9. Safety Considerations	1

9.1  Mains-Side Safety	1

9.2  Control-Side Safety	1

9.3  Enclosure & Environmental	1

10. Expansion Roadmap	1

11. Key References	1

Appendix A — Pilot Voltage Thresholds (Quick Reference)	1

Appendix B — UART Message Quick Reference	1

# 1. Introduction

An Electric Vehicle Supply Equipment (EVSE) unit — commonly called a charging station or wallbox — is the hardware that safely transfers AC power from the grid to an electric vehicle's on-board charger. This document covers the complete design, working principles, firmware architecture, and step-by-step implementation of a Level 2 EVSE built around two microcontrollers:

- STM32F103C8 ("Blue Pill") — safety core, IEC 61851 pilot signal, contactor control, fault detection

- ESP32-C6 — connectivity core, OCPP client, BLE authentication, OTA updates, web dashboard

This is a personal learning project intended to produce a functional, standards-aware charger. It is not a certified product. All mains-voltage work must comply with your local electrical codes and be performed or inspected by a qualified electrician.

ℹ NOTE This documentation assumes a Level 2 AC charging setup (single-phase 240 V, up to 32 A). Level 1 (120 V) operation follows the same logic at lower current ratings.

# 2. System Overview

## 2.1  Functional Block Diagram

The system is divided into two clearly separated domains: the high-voltage power path (mains, contactor, EVSE socket) and the low-voltage control plane (MCUs, sensors, communication).

┌─────────────────────────────────────────────────────────────────┐ │ HIGH-VOLTAGE POWER PATH │ │ │ │ L ──┬── Earth Leakage / RCCB ──── Contactor ──── EVSE J1772 │ │ N ──┘ │ ▲ │ │ PE ─────────────────────────────── Type-2 Socket │ └───────────────────────│──────────────────│──────────────────────┘ │ CT Sensor │ Relay drive ┌───────────────────────▼──────────────────▼──────────────────────┐ │ STM32F103C8 (Safety Core) │ │ │ │ • 1 kHz PWM pilot signal (±12 V via op-amp / relay) │ │ • State machine A → B → C → D │ │ • CT current sensing (ADC) │ │ • GFCI / over-current trip logic │ │ • Contactor coil drive (MOSFET / relay) │ │ • UART ←──────────────────────────────────────► ESP32-C6 │ └──────────────────────────────────────────────────────────────────┘ │ UART ┌─────────────────────────────────────▼──────────────────────────┐ │ ESP32-C6 (Connectivity Core) │ │ │ │ • OCPP 1.6J WebSocket client │ │ • Wi-Fi (STA + AP modes) │ │ • BLE 5 authentication / NFC-style tap │ │ • Local web dashboard (HTTP + JSON) │ │ • OTA firmware updates │ │ • MQTT energy telemetry (optional) │ └────────────────────────────────────────────────────────────────┘

## 2.2  Board Responsibilities Summary

| Function | Responsible Board / Notes |
| --- | --- |
| Pilot signal generation | STM32F103C8 — TIM3 PWM, external ±12 V conditioning circuit |
| IEC 61851 state machine | STM32F103C8 — interrupt-driven, <1 ms response |
| Contactor / relay control | STM32F103C8 — GPIO → MOSFET gate → coil |
| Current sensing (CT) | STM32F103C8 — ADC1, RMS calculation in firmware |
| GFCI / fault detection | STM32F103C8 — dedicated interrupt pin from RCCB auxiliary contact |
| OCPP 1.6J protocol | ESP32-C6 — MicroOCPP library, WebSocket over Wi-Fi |
| User authorisation (BLE) | ESP32-C6 + Fanstell BT832 (future expansion) |
| Web dashboard | ESP32-C6 — async HTTP server, JSON API |
| OTA firmware updates | ESP32-C6 — esp_https_ota, forwards update trigger to STM32 |
| Energy logging | ESP32-C6 — NVS flash ring buffer, syncs to OCPP backend |

# 3. Standards & Working Principle

## 3.1  Applicable Standards

| Standard | Relevance to this project |
| --- | --- |
| IEC 61851-1 | Electric vehicle conductive charging system — general requirements (pilot signal, states, control) |
| SAE J1772 | US/Canada connector standard for Level 1 & 2 AC charging — defines the same pilot protocol |
| IEC 62196-2 | Defines the Type 2 (Mennekes) connector geometry used in Europe |
| IEC 61008 | Residual current circuit breakers — underpins GFCI/RCD requirements |
| OCPP 1.6J | Open Charge Point Protocol — JSON over WebSocket, used for CSMS communication |
| NEC Article 625 | US National Electrical Code for EV charging equipment installation |

## 3.2  The IEC 61851 / J1772 Pilot Signal — In Depth

The Control Pilot (CP) line is the single wire that carries all handshake information between the EVSE and the vehicle. It works on three simultaneous channels encoded into one signal:

- Presence detection — whether a vehicle is physically connected

- State communication — vehicle readiness, ventilation request

- Current capability advertisement — the EVSE advertises its maximum available current via PWM duty cycle

### 3.2.1  Signal Characteristics

The EVSE generates a 1 kHz square wave on the CP line. The unloaded high level is +12 V and the low level is −12 V. When the vehicle connects, it loads the CP line with a specific resistor to ground through a diode, pulling the high level down to a voltage that the EVSE measures to determine vehicle state.

| State | Pilot V | PWM Duty | Meaning | Action |
| --- | --- | --- | --- | --- |
| A | +12 V DC | N/A (DC) | No vehicle — standby | Keep contactor open, advertise max current |
| B | +9 V | Any | Vehicle connected, not ready to charge | Contactor still open, wait for State C |
| C | +6 V | 10–96 % | Vehicle ready, requesting power | Close contactor, power flows to vehicle |
| D | +3 V | 10–96 % | Vehicle ready, requires ventilated area | Close contactor only if ventilation confirmed |
| E | 0 V | — | EVSE-side fault (short, GFCI trip) | Open contactor immediately |
| F | −12 V | — | EVSE fault / no pilot signal | Open contactor immediately |

### 3.2.2  Current Advertisement via PWM Duty Cycle

While in States B, C, or D, the EVSE modulates the duty cycle to tell the vehicle the maximum current it can draw:

| Duty Cycle | Formula | Max Current | Typical Use |
| --- | --- | --- | --- |
| 10 % | I = duty × 0.6 A | 6 A | Minimum, Level 1 |
| 16 % | I = duty × 0.6 A | 9.6 A | Common Level 1 |
| 25 % | I = duty × 0.6 A | 15 A | Level 2 low |
| 40 % | I = duty × 0.6 A | 24 A | Level 2 mid |
| 53 % | I = duty × 0.6 A | 32 A | Level 2 full (7.4 kW @ 230 V) |
| 85 % | I = (duty−64) × 2.5 | 80 A | >51 % uses alternate formula |

ℹ NOTE For this project, set duty = 40 % (24 A) as a safe starting point. The STM32 TIM3 ARR and CCR registers directly control this — details in Section 5.

## 3.3  Power Path

The mains supply (L, N, PE) enters the enclosure through a dedicated MCB (miniature circuit breaker) sized to the intended charge current, then passes through an RCD/RCCB for ground fault protection. The main contactor sits downstream of the RCD and is the only element that connects and disconnects power to the vehicle. The pilot state machine on the STM32 controls the contactor coil — no software fault can bypass the physical break.

⚠ WARNING The contactor must be rated for AC-3 duty at the full load current. For a 32 A, 240 V installation, use a 40 A AC contactor minimum. Never use a DC relay for this purpose.

# 4. Hardware Design

## 4.1  Bill of Materials

| Component | Specification | Qty | Notes |
| --- | --- | --- | --- |
| STM32F103C8 (Blue Pill) | 72 MHz ARM Cortex-M3 | 1 | Safety core MCU |
| ESP32-C6 DevKit | Wi-Fi 6, BLE 5, 802.15.4 | 1 | Connectivity core |
| AC Contactor | 25–40 A, 240 V AC coil | 1 | Schneider LC1D or equiv. |
| RCCB / RCD | 2-pole, 30 mA, Type A | 1 | Earth fault protection |
| MCB | 2-pole, 32 A, Curve C | 1 | Upstream protection |
| CT Sensor | SCT-013-030 or SCT-013-000 | 1 | Non-invasive, 30 A |
| ±12 V PSU module | Small split-rail for pilot circuit | 1 | e.g. RECOM RPM5.0 |
| 5 V / 3.3 V PSU | DIN-rail or PCB-mount | 1 | Powers MCUs & logic |
| Op-Amp (MCP6002) | Rail-to-rail, SOT-23 | 1–2 | Pilot buffer / comparator |
| MOSFET (IRLZ44N) | Logic-level N-channel | 1 | Contactor coil drive |
| J1772 / Type-2 Socket | IEC 62196-2 inlet | 1 | Vehicle-facing connector |
| TVS Diode (P6KE15A) | Bidirectional, 15 V | 1 | Pilot line transient protection |
| Resistors — pilot circuit | 880 Ω, 1.3 kΩ, 270 Ω (1 %) | 1 set | State detection divider |
| Enclosure (IP54 min.) | DIN-rail capable, outdoor rated | 1 | Rittal or equiv. |
| Type-B RCCB (optional) | Detects DC residual current | 1 | For 3-phase/DC-coupled EVs |

## 4.2  Pilot Signal Conditioning Circuit

The STM32 GPIO operates at 3.3 V, but the pilot signal requires ±12 V levels. An external circuit translates between these domains. The standard approach uses a comparator or op-amp based level shifter with two back-to-back zener diodes clamping the output, plus a resistor divider to scale the return measurement back to 0–3.3 V for the STM32 ADC.

STM32 PA1 (PWM) ──── 1 kΩ ─────┬──── CP Line (to vehicle) │ +12 V ──────────────────────────┤ (pulled to +12 V through 1 kΩ when pilot high) │ −12 V ──── 1N4148 ──────────────┘ (negative swing on low phase) │ [Vehicle loads this line] State B: 2.74 kΩ to PE → +9 V Measurement side: State C: 882 Ω to PE → +6 V State D: 246 Ω to PE → +3 V │ CP ──── 6.8 kΩ ──── STM32 PA0 (ADC) ← scaled to 0–3.3 V └── 3.3 kΩ ──── GND

ℹ NOTE The ADC measurement must only be sampled during the positive (high) phase of the PWM cycle. Trigger your ADC conversion from TIM3's capture-compare event to ensure timing alignment.

## 4.3  Current Sensing Circuit

A split-core CT (SCT-013-000, 100 A : 50 mA) is clamped around one of the live conductors. The secondary current flows through a burden resistor (chosen per CT datasheet — typically 33 Ω for 100 A full scale) to produce a proportional AC voltage. A biasing circuit shifts this AC signal to sit in the middle of the STM32's 0–3.3 V ADC range (bias at 1.65 V).

CT Secondary ─────── Burden R (33 Ω) ─────┬──── PA2 (ADC) │ 3.3 V ── 10 kΩ ──┬── 10 kΩ ── GND │ └──────────────────────────┘ (1.65 V bias) RMS calculation in firmware: 1. Sample at 2 kHz for 50 ms (one full cycle at 50 Hz) 2. Remove DC bias: subtract rolling average 3. Square → accumulate → average → square root 4. Scale by CT ratio and burden R to get amperes

## 4.4  STM32 Pin Assignment

| Pin | Function |
| --- | --- |
| PA1 | TIM2_CH2 — PWM output for pilot signal (1 kHz) |
| PA0 | ADC1_IN0 — Pilot voltage measurement (state detection) |
| PA2 | ADC1_IN2 — CT sensor current measurement |
| PA3 | ADC1_IN3 — Temperature sensor (NTC, optional) |
| PB0 | GPIO Output — Contactor drive (to MOSFET gate) |
| PB1 | GPIO Output — Status LED (green = charging) |
| PB10 | USART3_TX — UART to ESP32-C6 |
| PB11 | USART3_RX — UART from ESP32-C6 |
| PB12 | GPIO Input — GFCI/RCD auxiliary contact (interrupt) |
| PB13 | GPIO Input — Emergency stop / door open (interrupt) |
| PB14 | GPIO Output — Error LED (red) |
| PB15 | GPIO Output — Fault relay (cuts pilot on hard fault) |
| PC13 | GPIO Output — Onboard LED (heartbeat) |

## 4.5  ESP32-C6 Pin Assignment

| Pin | Function |
| --- | --- |
| GPIO4 | UART TX → STM32 USART3 RX |
| GPIO5 | UART RX ← STM32 USART3 TX |
| GPIO6 | Status LED |
| GPIO7 | User button (factory reset / BLE pairing mode) |
| GPIO18 | I2C SDA — optional OLED display |
| GPIO19 | I2C SCL — optional OLED display |

# 5. STM32F103C8 Firmware

## 5.1  Toolchain Setup

Use STM32CubeIDE (free, Eclipse-based) with the HAL library. Alternatively, use arm-none-eabi-gcc with libopencm3 for a bare-metal approach. This document uses STM32CubeIDE + HAL + FreeRTOS.

- Download and install STM32CubeIDE from st.com

- Create new project: File → New → STM32 Project → select STM32F103C8Tx

- In the .ioc file (CubeMX), configure peripherals as per Section 4.4

- Enable FreeRTOS (CMSIS-RTOS v2) under Middleware

- Enable USART3 in asynchronous mode, 115200 baud, DMA both directions

- Enable TIM2 Channel 2 in PWM output mode

- Enable ADC1 with channels IN0 and IN2, scan mode, DMA

- Generate code, then implement task logic in the generated freertos.c

## 5.2  Pilot PWM Configuration

TIM2 runs from the 72 MHz APB1 timer clock (after prescaler). Configure it for a 1 kHz output with a duty cycle that matches the desired current advertisement.

/* In MX_TIM2_Init() — generated by CubeMX, edit PSC and ARR */ htim2.Init.Prescaler = 71; // 72 MHz / (71+1) = 1 MHz timer clock htim2.Init.CounterMode = TIM_COUNTERMODE_UP; htim2.Init.Period = 999; // ARR: 1 MHz / 1000 = 1 kHz htim2.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1; /* Duty cycle helper — call this when current limit changes */ void pilot_set_current_amps(uint8_t amps) { uint32_t duty_pct; if (amps >= 6 && amps <= 51) { duty_pct = (uint32_t)amps * 10 / 6; // duty = amps / 0.6 } else if (amps > 51 && amps <= 80) { duty_pct = (uint32_t)(amps / 2.5) + 64; } else { duty_pct = 0; // disable } uint32_t ccr = (duty_pct * 1000) / 100; // scale to ARR=999 __HAL_TIM_SET_COMPARE(&htim2, TIM_CHANNEL_2, ccr); } /* Initialise at 24 A (40% duty) */ HAL_TIM_PWM_Start(&htim2, TIM_CHANNEL_2); pilot_set_current_amps(24);

## 5.3  Pilot State Machine

The state machine runs in its own FreeRTOS task at 10 ms period. It reads the ADC-measured pilot voltage and drives the contactor output accordingly.

typedef enum { EVSE_STATE_A = 0, // No vehicle EVSE_STATE_B, // Vehicle connected, not ready EVSE_STATE_C, // Charging EVSE_STATE_D, // Charging, ventilation required EVSE_STATE_E, // Error / fault } EvseState_t; static EvseState_t current_state = EVSE_STATE_A; /* Called from PilotTask every 10 ms */ void pilot_state_machine_tick(void) { float v = pilot_read_voltage_v(); // ADC → scaled to volts EvseState_t new_state; if (v > 10.5f) new_state = EVSE_STATE_A; else if (v > 7.5f && v <= 10.5f) new_state = EVSE_STATE_B; else if (v > 4.5f && v <= 7.5f) new_state = EVSE_STATE_C; else if (v > 1.5f && v <= 4.5f) new_state = EVSE_STATE_D; else new_state = EVSE_STATE_E; if (new_state != current_state) { evse_on_state_change(current_state, new_state); current_state = new_state; } } void evse_on_state_change(EvseState_t from, EvseState_t to) { switch (to) { case EVSE_STATE_A: contactor_open(); break; case EVSE_STATE_B: contactor_open(); // wait for vehicle ready break; case EVSE_STATE_C: if (authorization_granted()) { contactor_close(); } break; case EVSE_STATE_D: /* Only close if ventilation is available */ /* For indoor residential use, treat D as a fault */ contactor_open(); report_fault(FAULT_VENTILATION_REQUIRED); break; case EVSE_STATE_E: contactor_open(); report_fault(FAULT_PILOT_ERROR); break; } uart_send_state_to_esp32(to); // notify ESP32 }

## 5.4  Fault Detection

Two fault sources require immediate contactor opening regardless of pilot state:

- GFCI / RCD trip — the RCD auxiliary contact changes state; this is wired to PB12 as an EXTI interrupt

- Over-current — the CT measurement exceeds 110 % of the set point for more than 100 ms

/* GFCI interrupt handler — fires on PB12 falling edge */ void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin) { if (GPIO_Pin == GFCI_Pin) { contactor_open(); current_state = EVSE_STATE_E; pilot_disable_pwm(); uart_send_fault_to_esp32(FAULT_GFCI_TRIP); /* Latch: require manual reset */ gfci_latched = true; } } /* Over-current check inside MeteringTask (100 ms period) */ void metering_check_overcurrent(void) { float i_rms = ct_read_rms_amps(); float limit = pilot_get_current_limit_amps() * 1.1f; if (i_rms > limit) { oc_timer_ms += 100; if (oc_timer_ms >= 100) { contactor_open(); report_fault(FAULT_OVERCURRENT); } } else { oc_timer_ms = 0; } }

## 5.5  FreeRTOS Task Layout

| Task / Module | Description |
| --- | --- |
| PilotTask | Priority 5 (high) — 10 ms period. Reads pilot ADC, runs state machine, drives contactor. |
| MeteringTask | Priority 3 — 100 ms period. Computes CT RMS, checks over-current, accumulates energy (kWh). |
| CommsTask | Priority 2 — 50 ms period. Serialises telemetry to JSON, sends over USART3 DMA to ESP32. |
| HeartbeatTask | Priority 1 (lowest) — 500 ms period. Toggles PC13 LED. If this stops, WDT resets the MCU. |
| IWDG | Independent watchdog — 2 s timeout. Pet inside HeartbeatTask. Resets MCU on lockup. |

## 5.6  UART Protocol (STM32 → ESP32)

Messages are newline-terminated JSON strings at 115200 baud. This keeps the protocol human-readable and easy to debug with a serial monitor.

/* STM32 → ESP32 (sent every 500 ms or on state change) */ { "t": "status", "state": "C", "i_rms": 15.3, "kwh": 4.72, "fault": 0, "limit_a": 24 } /* ESP32 → STM32 (command) */ { "t": "cmd", "cmd": "set_limit", "val": 16 } /* ESP32 → STM32 (authorisation) */ { "t": "auth", "granted": true, "id_tag": "ABC12345" }

# 6. ESP32-C6 Firmware

## 6.1  Development Environment

Use ESP-IDF v5.x (not Arduino). ESP-IDF gives you direct FreeRTOS control, proper partition table management, and full access to the OCPP library's build system.

- Install ESP-IDF: follow espressif.com/en/getting-started

- Clone MicroOCPP: git clone https://github.com/matth-x/MicroOcpp

- Add MicroOCPP as a component in your project's components/ directory

- Configure via menuconfig: idf.py menuconfig

- Set Wi-Fi credentials in NVS (not in source code — use nvs_set_str at first boot)

## 6.2  FreeRTOS Task Layout

| Task | Description |
| --- | --- |
| UartRxTask | Priority 5 — UART receive from STM32. Parses JSON, updates shared state struct. |
| OcppTask | Priority 4 — MicroOCPP loop(). Handles WebSocket ping/pong, meter values, transactions. |
| HttpTask | Priority 3 — Async HTTP server. Serves dashboard HTML + JSON API at /api/status. |
| BleAuthTask | Priority 2 — BLE GATT server. Listens for authorisation tokens from phone/fob. |
| OtaTask | Priority 1 — Checks OTA endpoint daily. Downloads and flashes update if available. |

## 6.3  OCPP 1.6J Integration

MicroOCPP handles the entire OCPP state machine. You register callbacks for the actions your hardware performs, and the library calls them at the right time.

#include "MicroOcpp.h" void ocpp_init(void) { mocpp_initialize( "wss://your-csms.example.com/ocpp", // CSMS WebSocket URL "EVSE-001", // Charge Point ID "My EVSE", // Display name "YourModel", // Model "v1.0" // Firmware version ); /* Tell OCPP how to read energy meter */ mocpp_setEnergyMeterInput([]() -> float { return (float)shared_state.kwh * 1000.0f; // return Wh }); /* Tell OCPP how to read current */ mocpp_setCurrentSampledValueInput([]() -> float { return shared_state.i_rms; }); /* OCPP calls this when a transaction should start */ mocpp_setConnectorPluggedInput([]() -> bool { return shared_state.evse_state == 'C' ¦¦ shared_state.evse_state == 'B'; }); /* OCPP calls this to enable/disable charging */ mocpp_setEvReadyInput([]() -> bool { return shared_state.evse_state == 'C'; }); } /* Call mocpp_loop() inside OcppTask at ≥100 ms intervals */ void ocpp_task(void *pvParameters) { ocpp_init(); for (;;) { mocpp_loop(); vTaskDelay(pdMS_TO_TICKS(100)); } }

## 6.4  BLE Authorisation

The ESP32-C6 runs a simple GATT server with one custom service. A phone app or BLE fob writes an ID tag to the characteristic; the ESP32 validates it (locally or against OCPP RemoteStartTransaction) and sends the result to the STM32 over UART.

/* Custom BLE service UUIDs */ #define SVC_UUID "4fafc201-1fb5-459e-8fcc-c5c9c331914b" #define CHAR_UUID "beb5483e-36e1-4688-b7f5-ea07361b26a8" /* On BLE write callback */ void ble_on_id_tag_received(const char *id_tag) { /* Simple local whitelist check */ if (nvm_whitelist_contains(id_tag)) { uart_send_auth_to_stm32(true, id_tag); mocpp_beginTransaction(id_tag); } else { /* Forward to CSMS via OCPP Authorize.req */ mocpp_authorize(id_tag, [](JsonObject response) { bool granted = (strcmp(response["idTagInfo"]["status"], "Accepted") == 0); uart_send_auth_to_stm32(granted, id_tag); }); } }

## 6.5  Local Web Dashboard

The HTTP server hosts a single-page dashboard. A JavaScript fetch() polls /api/status every 2 seconds to update the display without page refresh.

/* GET /api/status — returns current system state as JSON */ { "state": "C", "i_rms": 15.3, "v_rms": 230.0, "power_kw": 3.52, "energy_kwh": 4.72, "session_kwh": 1.38, "session_min": 23, "fault": null, "ocpp_connected": true, "id_tag": "ABC12345" } /* POST /api/command — send commands from dashboard */ { "cmd": "set_limit", "val": 16 }

# 7. Step-by-Step Implementation

## Phase 1 — Bench Validation (No Mains)

Complete these steps on the bench with 5 V / 3.3 V bench supplies only. Do not connect mains at this stage.

- Build the pilot conditioning circuit on breadboard. Power from ±12 V bench supply.

- Connect a 2.74 kΩ resistor to ground to simulate a State-B vehicle. Verify the STM32 ADC reads ~9 V equivalent.

- Implement and flash the pilot state machine firmware. Confirm state transitions via UART debug output.

- Replace the 2.74 kΩ with 882 Ω (State C). Confirm contactor drive GPIO goes high (do not connect actual contactor yet — use LED as proxy).

- Connect STM32 USART3 to ESP32-C6. Verify JSON telemetry messages are received correctly.

- Bring up ESP32-C6 Wi-Fi. Connect to your CSMS test server (use SteVe or a free cloud OCPP endpoint). Verify BootNotification and Heartbeat.

- Test BLE authorisation from a phone. Verify the ESP32 sends the auth JSON to the STM32 and the contactor LED activates.

## Phase 2 — Power Path Integration (Low Voltage First)

Integrate the contactor and current sensing on the bench, still without mains.

- Wire the contactor coil to the STM32 MOSFET drive circuit. Use a 12 V bench supply for the coil — NOT mains.

- Verify the contactor physically opens and closes on state changes. Check flyback diode across coil is correctly installed.

- Install the CT sensor around a wire. Run 5 A from a bench load through it. Verify the STM32 ADC reads the correct RMS value.

- Implement and verify the over-current trip: set limit to 5 A, push 6 A through the CT wire, confirm contactor opens within 100 ms.

- Wire PB12 to a push button simulating a GFCI trip. Confirm contactor opens immediately and fault latches.

## Phase 3 — Full Integration (Qualified Electrician Required)

This phase involves mains voltage. All wiring must be done by or inspected by a qualified electrician. Follow your local wiring regulations.

- Mount MCB, RCD, contactor, and enclosure. Have wiring inspected before energising.

- Connect mains L, N, PE through MCB → RCD → contactor → Type-2 socket.

- Wire RCD auxiliary contact to STM32 PB12.

- Power up with MCB only (contactor open). Verify 5 V / 3.3 V PSU rails are correct.

- With no vehicle connected, verify State A and that the contactor remains open.

- Connect a J1772 / Type-2 test plug with correct resistors (bench test plug). Verify states B and C transitions.

- Connect a real EV. Authorise via BLE. Verify charging starts and current is measured correctly.

- Monitor via local dashboard and OCPP backend. Verify StopTransaction on vehicle disconnect.

## Phase 4 — System Hardening

- Enable and test the Independent Watchdog on the STM32 (IWDG). Pull the heartbeat task's pet call — confirm MCU resets within 2 s.

- Test OTA update: build a new ESP32 firmware image, host it, trigger update from dashboard. Confirm successful flash and reboot.

- Soak test: charge a real EV through a full session. Log energy data, verify kWh accuracy against a reference meter.

- Simulate CSMS disconnection: verify the charger continues to operate in offline mode and syncs session data on reconnection.

- Document your final calibration values (CT burden R, ADC scale factors, pilot voltage thresholds) in a commissioning record.

# 8. Testing & Validation

## 8.1  Test Matrix

| Test | Method | Pass Criterion |
| --- | --- | --- |
| State A detection | No vehicle, measure CP voltage | +12 V ± 0.5 V on CP |
| State B detection | Plug in 2.74 kΩ resistor to CP | +9 V ± 0.5 V, contactor open |
| State C detection | Plug in 882 Ω resistor to CP | +6 V ± 0.5 V, contactor closes after auth |
| Current limit 16 A | Set limit, verify PWM duty = 26.7 % | Oscilloscope duty 26.7 % ± 0.5 % |
| Current limit 24 A | Set limit, verify PWM duty = 40 % | Oscilloscope duty 40 % ± 0.5 % |
| GFCI trip response | Simulate RCD aux contact open | Contactor opens < 50 ms, fault latched |
| Over-current trip | CT overcurrent > 110 % for 200 ms | Contactor opens, fault sent to ESP32 |
| Watchdog reset | Remove IWDG pet, measure reset time | MCU resets within 2 s |
| OCPP BootNotification | Cold start, monitor WebSocket traffic | Accepted response within 10 s |
| OCPP StartTransaction | Authorise, connect vehicle | StartTransaction.conf received |
| OCPP MeterValues | Active session, check CSMS log | MeterValues sent at configured interval |
| OCPP StopTransaction | Disconnect vehicle | StopTransaction sent, energy correct |
| OTA update | Host new image, trigger from dashboard | New firmware running after reboot |
| BLE authorisation | Scan and write ID tag via phone | Charging starts within 2 s of write |
| Offline resilience | Disconnect CSMS during charge | Charge continues, data syncs on reconnect |

# 9. Safety Considerations

⚠ WARNING This section is not optional reading. A poorly built EVSE can cause electrocution, fire, or vehicle damage. Read every point before handling mains voltage.

## 9.1  Mains-Side Safety

- Always isolate the supply at the MCB before working inside the enclosure.

- The contactor must be the only point in the software-controlled path. Never rely on a MOSFET or relay alone for the power break.

- The RCD must be Type A minimum (detects pulsating DC fault current). For 3-phase or DC-coupled EVs, use Type B.

- All mains-side wiring must be rated for the full load current plus 25 % margin.

- The PE (protective earth) must be continuous and measured with a loop impedance tester before commissioning.

- Install surge protection (SPD) on the mains input if the installation is exposed to lightning.

## 9.2  Control-Side Safety

- The pilot conditioning circuit must use proper isolation if built on the same PCB as the MCU. Keep at least 6 mm creepage distance between ±12 V pilot traces and 3.3 V logic.

- The GFCI/RCD trip must be hardware-wired to cut the contactor coil supply, in addition to the STM32 interrupt. Software alone is not a safety barrier.

- The watchdog must be enabled at all times in production firmware. A locked-up MCU with the contactor closed is a fire risk.

- Never store Wi-Fi passwords or authorisation tokens in plaintext in source code. Use NVS with encryption enabled.

## 9.3  Enclosure & Environmental

- Use an IP54 minimum rated enclosure for any outdoor or garage installation.

- Provide adequate ventilation or thermal management if the contactor and power supply are enclosed together.

- Label the enclosure clearly: voltage, current rating, and emergency isolation point.

# 10. Expansion Roadmap

| Feature | Implementation Path |
| --- | --- |
| Precision energy metering | Add ATM90E32 SPI metering IC. Replaces CT + STM32 ADC approach with utility-grade ±0.1 % accuracy. |
| RFID authorisation | Connect MFRC522 SPI reader to ESP32. Issue and manage RFID cards locally without BLE dependency. |
| Load management | Read whole-home current via second CT. Dynamically reduce EVSE current limit to prevent main fuse trip. |
| Solar integration | Read inverter output via Modbus TCP or SunSpec REST. Prioritise charging when solar surplus is available. |
| OCPP 2.0.1 upgrade | MicroOCPP supports 2.0.1. Enables smart charging profiles, ISO 15118 Plug & Charge groundwork. |
| Fanstell BT832 auth fob | Pair the BT832 as a dedicated BLE keyfob. Program it to advertise a fixed authorisation UUID on button press. |
| STM32H757 migration | The STM32F103 firmware is HAL-compatible. Port to H757 for dual-core operation, better ADC, and CAN bus. |
| OLED status display | Connect 0.96″ SSD1306 over I2C to ESP32. Show state, current, kWh, and session duration locally. |

# 11. Key References

| Resource | Notes / URL |
| --- | --- |
| IEC 61851-1 (2017) | Electric vehicle conductive charging system. Available via iec.ch |
| SAE J1772 (2017) | SAE Electric Vehicle and Plug in Hybrid Electric Vehicle Conductive Charge Coupler |
| MicroOCPP library | github.com/matth-x/MicroOcpp — OCPP 1.6J/2.0.1 for embedded systems |
| SteVe CSMS | github.com/steve-community/steve — open-source OCPP Central System for testing |
| ESP-IDF documentation | docs.espressif.com/projects/esp-idf |
| STM32CubeIDE | st.com/stm32cubeide |
| OpenEVSE project | openevse.com — open-source EVSE reference design |
| EVduino / EVSE-WiFi | Community firmware references for J1772 pilot signal implementation |

# Appendix A — Pilot Voltage Thresholds (Quick Reference)

| State | Pilot V | PWM Duty | Meaning | Action |
| --- | --- | --- | --- | --- |
| A | +12 V ± 0.5 | N/A | No vehicle present | Pilot ON, contactor OPEN |
| B | +9 V ± 0.5 | Any | Vehicle connected | Contactor OPEN, await auth |
| C | +6 V ± 0.5 | 10–96 % | Ready to charge | Close contactor (if authorised) |
| D | +3 V ± 0.5 | 10–96 % | Needs ventilation | Treat as fault for residential |
| E | 0 V | — | Pilot shorted / EVSE fault | Open contactor, latch fault |
| F | −12 V | — | No pilot / EVSE fault | Open contactor, latch fault |

# Appendix B — UART Message Quick Reference

Direction Type Key fields ────────────────────────────────────────────────────────── STM→ESP status state, i_rms, kwh, fault, limit_a STM→ESP fault code, description ESP→STM auth granted (bool), id_tag (string) ESP→STM cmd cmd: set_limit ¦ stop ¦ reset_fault val: integer (for set_limit: amps) All messages: newline-terminated JSON, 115200 8N1 Max length: 256 bytes STM32 timeout: if no ESP heartbeat for 10 s, continue standalone

— End of Document —